# Product Catalog

Specifications below are used by sales/customer service staff to answer compatibility questions.

# Mixer Taps

## PC-101 — Aria Basin Mixer

Type: Single-lever basin mixer, chrome finish

Minimum operating pressure: 50 kPa (works on low-pressure/gravity-fed systems)

Maximum operating pressure: 800 kPa

Recommended for: gravity-fed hot water cylinders, low-pressure council mains

Material: Brass body, ceramic disc cartridge

Warranty: 15 years on body, 5 years on cartridge

## PC-102 — Vale Kitchen Mixer with Pull-Out Spray

Type: Single-lever kitchen mixer

Minimum operating pressure: 150 kPa

Maximum operating pressure: 1000 kPa

Recommended for: mains-pressure systems only — not suitable for low-pressure gravity-fed cylinders

Material: Brass body, stainless steel braided hoses

Warranty: 10 years on body, 2 years on spray mechanism

## PC-103 — Coastal Bath Mixer (Wall Mounted)

Type: Wall-mounted bath spout mixer

Minimum operating pressure: 50 kPa

Maximum operating pressure: 600 kPa

Recommended for: both mains and low-pressure systems

Material: Brass body, chrome finish

Warranty: 15 years on body

# Showers

## SH-201 — Rainfall Overhead Shower Set (200mm head)

Minimum operating pressure: 50 kPa

Note: below 100 kPa, flow rate will be noticeably reduced — recommend pairing with a low-pressure-rated mixer (PC-101 or PC-103)

Maximum operating pressure: 500 kPa

Material: Stainless steel head, brass arm

Warranty: 10 years

## SH-202 — Adjustable Slide Shower Set

Minimum operating pressure: 150 kPa

Maximum operating pressure: 1000 kPa

Requires mains pressure — not recommended for gravity-fed systems

Material: Chrome-plated brass

Warranty: 10 years

## SH-203 — Twin Shower (Overhead + Handheld)

Minimum operating pressure: 200 kPa

Maximum operating pressure: 1000 kPa

Requires mains pressure and a diverter valve (sold separately, code PC-110)

Warranty: 10 years on fittings, 2 years on diverter valve

# Toilets & Cisterns

## TL-301 — Compact Close-Coupled Toilet Suite

Cistern type: Dual-flush (3L/6L)

Minimum inlet pressure: 35 kPa

Maximum inlet pressure: 1000 kPa

Pan-to-wall clearance required: minimum 650mm

Warranty: 10 years on ceramic, 5 years on cistern mechanism

## TL-302 — Wall-Faced Toilet Suite (In-Wall Cistern)

Cistern type: Dual-flush (2.5L/4L), in-wall unit

Minimum inlet pressure: 50 kPa

Maximum inlet pressure: 1000 kPa

Requires wall frame installed prior to wall lining (see installation-guides)

Warranty: 10 years on ceramic, 10 years on in-wall cistern frame

# Accessories & Valves

## PC-110 — 2-Way Shower Diverter Valve

Minimum operating pressure: 150 kPa

Used with SH-203 twin shower systems

Material: Brass, chrome finish

Warranty: 5 years

## PC-111 — Pressure-Limiting Valve (PLV)

Reduces incoming mains pressure to a safe working range

Adjustable output: 250–500 kPa

Recommended where incoming mains pressure exceeds 1000 kPa

Warranty: 5 years

## PC-112 — Water Hammer Arrestor

Fits inline on 15mm and 20mm supply lines

Reduces pipe noise/hammering from quick-closing valves (e.g. washing machine solenoids)

Warranty: 3 years

# Notes for Staff

"Low-pressure" or "gravity-fed" typically refers to systems below 150 kPa — common in older homes with a header tank in the roof space.

"Mains pressure" systems are typically 250–500 kPa in most residential areas, but can be checked against council supply data if a customer is unsure.

If a customer doesn't know their water pressure, recommend the PC-101, PC-103, or SH-201 as safe low-pressure-compatible defaults, or advise a plumber test the pressure on-site before purchase.